National University of Life and Environmental

Sciences of Ukraine

Department of Machines and Equipment Design

Practice Book

„Constructing Machines”

Student:___________________

Academic group:____________

Faculty:___________________

__________________________

Instructor: Romasevych Yuriy

Kyiv - 2021

Рекомендовано до друку вченою радою факультету конструювання та дизайну. Протокол № ___  від ___________2021 р.

Укладач:

Ромасевич Юрій Олександрович

Рецензенти:

Афтанділянц Євгеній Григорович, доктор технічних наук, професор, завідувач кафедрою технології конструкційних матеріалів і матеріалознавства НУБіП України;

Пилипенко Андрій Петрович, кандидат технічних наук, доцент, доцент кафедри механіки НУБіП України.

У робочому зошиті наведено опис лабораторних установок, необхідні довідкові дані, представлено таблиці звітів, методику та порядок виконання лабораторних робіт із дисципліни «Будівельна техніка». Англійською мовою.

Робочий зошит призначений для студентів, які проходять підготовку за спеціальністю 192 – «Будівництво і цивільна інженерія».

НАВЧАЛЬНЕ ВИДАННЯ

© Ромасевич Ю.О., 2021

© НУБІП України

The order of the laboratory classes conducting

During the study of the course the instructors of the department familiarize the students with the plans of conducting laboratory classes, the rules for labor protection in the laboratory, organization of working time and the procedure for conducting laboratory work, drawing the reports of completed of laboratory classes i.e.

Considerable attention must be paid to the study of the peculiarities of work in the laboratory, elements of scientific research during laboratory classes. The first lesson ends with a safety briefing. It must be written in a special journal.

During preparing for the laboratory classes, each student must have profound knowledge of the safety rules when handling the equipment and be able to provide first aid to his coworkers in the event of an injury or electric shock.

Students obtain biggest effect from laboratory classes in the case of work with a team of 3 to 6 people. The division of the academic group into the brigade has to be carried out during the first class. At the same class students receive methodical literature and a list of recommended literature as well. Until the next laboratory class students must prepare themselves in advance and individually. They should familiarize themselves with the relevant sections of the course, study the methodological guidelines, write down the purpose of the work in the practice book and prepare the appropriate tables for the records and calculation data.

Reports on laboratory classes must be recorded in the practice book. On the cover of the practice book one must write his/her name and initials, as well as the faculty, course, academic group.

Before the beginning of a laboratory class, the teacher has to check the execution of previous classes’ reports, and the quality of preparing of the students for the current class. Unripe students are not allowed to work in the laboratory.

Experimental studies are conducted by students fully in accordance with the methodological recommendations given by the teacher.

After completion of the laboratory studies the obtained results should be shown to the instructor. If the results of the experiments are considered unsatisfactory, the laboratory class should be repeated. After the work is completed, the workplace should be prepared to the next classes. The team must gently remove tools and appliances.

Processing of experimental and calculation data can be performed in the laboratory or partly at home.

The execution of the reports should be made taking into account the requirements of the approved standards.

The reports are individually defended by each student during the subsequent laboratory classes, during consultations or in the hours that are scheduled for independent work.

Missed laboratory work must be carried out in accordance with the schedule and in agreement with the instructor who conducts the classes.

Part One

Conveying Machines

Belt Conveyor

Subject: the study and parameters calculation of a belt conveyor.

Purpose: to study the structure and peruse of the belt conveyors operating principle; to determine the main parameters of the conveyor (productivity, length of transportation, speed of the load).

Order of execution:

1. Draw up the scheme of the belt conveyor offered by the instructor for studying. Show main parts of the conveyor

2. Thoroughly study the proposed laboratory installation and perform measurements of structural dimension and set them into the table:

| Parameter name | Notation | Unit of measurement | Value | Note |
| --- | --- | --- | --- | --- |
| Structural elements |  |  |  |  |
| Width of the belt | В | m |  | measure |
| Thickness of the belt | δ | mm |  | measure |
| Length of the belt | l | m |  | measure |
| Diameter of the drive drum | dδ1 | m |  | measure |
| Diameter of the tension drum | dδ2 | m |  | measure |
| Diameter of the supporting rollers | dρ | m |  | measure |
| Diameter of the deflecting drum | dδ3 | m |  | measure |
| Drive |  |  |  |  |
| Rated speed of the engine | ωen | rad/s |  | determine by observation |
| Gear box ratio | Ug.b. |  |  | calculate |
| Reduction ratio of the chain drive | Uch |  |  | calculate |
| Reduction ratio of the belt drive | Ub |  |  | calculate |
| Number of teeth of the chain drive sprocket | z1 |  |  | count |
| Number of teeth of the chain driven sprocket | z2 |  |  | count |
| Diameter of the drive sheave of the belt drive | d1 | m |  | measure |
| Diameter of the driven sheave of the belt drive | d2 | m |  | measure |
| General drive ratio | U |  |  | calculate |
| Angular rate | ω1 | rad/s |  | calculate |
| Main conveyor characteristics |  |  |  |  |
| Lineal speed of the belt | V | m/s |  | measure |
| Specific gravity of the material to convey* | γ | kg/m3 |  | in appen-dices |
| Conveyor productivity | Q | kg/s |  | calculate |

*Instructor suggests the material to convey

2. Make conclusions about the design of the belt conveyor elements, which is considered during the laboratory class (the belt tension method, the complexity of the drive, adjusting of the belt, unloading of the belt conveyor etc.).

3. Make the draft of the belt ends joint

4. Show the result of calculations of conveyor productivity

Laboratory class conclusions with suggestions about improvement of belt conveyors operation:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Screw Conveyor

Subject: the study and parameters calculation of a screw conveyor.

Purpose: to study the structure and peruse of the screw conveyors operating principle; to determine the main parameters of the conveyor (productivity and power of the conveyor’s drive).

Order of execution:

1. Draw up the kinematic scheme of the screw conveyor offered by the instructor for studying. Show main parts of the conveyor.

2. Learn the screw conweyor structure and set its main charackteristics to the table above.

| Parameter name | Notation | Unit of measurement | Value | Note |
| --- | --- | --- | --- | --- |
| Structural elements |  |  |  |  |
| Type of the screw | - | - |  | determine by observation |
| Direction of the screw | - | - |  | determine by observation |
| External diameter of the screw | D | m |  | measure |
| Pitch of the screw | ρ | m |  | measure |
| Length of the screw | L | m |  | measure |
| Number of bearings | - | - |  | count |
| Number of intermediate bearings | - | - |  | count |
| Diameter of the shaft | d | m |  | measure |
| Drive |  |  |  |  |
| Engine power rating | Pr | kWt |  | determine by observation |
| Rated speed of the engine | ωen | rad/s |  | determine by observation |
| Reduction ratio of: - gear box - belt drive | U | - |  | calculate |
| Main conveyor characteristics |  |  |  |  |
| Specific gravity of the material to convey* | γ | kg/m3 |  | in appen-dices |
| Rate speed of the screw | ωsc | rpm |  | calculate |
| Critical rate speed of the screw | ωcr | rpm |  | calculate |
| Filling coefficient of the interturn space | ψ | - |  | determine |
| Coefficient which take into account the reducing of productivity caused by space between screw and the cover | c | - |  | determine |
| Conveyor inclination | β | deg |  | determine |
| Coefficient which take into account the reducing of productivity caused by the conveyor inclination | c0 | - |  | determine |
| Conveyor productivity | Q | kg/s |  | calculate |

*Instructor suggests the material to convey

3. Make an esquisse of screw disc with measures

4. Draw conclusions with the result of the experiments (advantages and disadvantages of screw conveyors)

5. Show the result of calculations of conveyor productivity

Laboratory class conclusions with suggestions about improvement of screw conveyors operation:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Bucket Conveyor

Subject: the study and parameters calculation of a bucket conveyor.

Purpose: to study the structure and main elements of the bucket conveyors loading and unloading methods; to determine the main parameters of the conveyor.

Order of execution:

1. Draw up the kinematic scheme of the bucket conveyor offered by the instructor for studying. Show main parts of the conveyor.

2. Subscribe the bucket conveyor construction (type of the pulling element, type of the bucket, type of the tension unit, type of the loading, elements of the drive i.e.)

3. Make an esquisse of a bucket with dimensions

4. Determine the main construction parameters of the bucket conveyor and fill the table above.

| Parameter name | Notation | Unit of measurement | Value | Note |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| Structural elements |  |  |  |  |
| Width of the belt | В | m |  | measure |
| Thickness of the belt | δ | mm |  | measure |
| Diameter of the drive drum | d | m |  | measure |
| Volume of a bucket | і | m3 |  | calculate |
| Bucket step | a | m |  | measure |
| Filling coefficient | ψ | - |  | select |
| Drive |  |  |  |  |
| Rated speed of the geared motor | ω | rad/s |  | determine by observation |

Continuation of the table

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Diameter of the drive-shaft | D | mm |  | measure |
| Engine power rating | P | kWt |  | determine by observation |
| Main conveyor characteristics |  |  |  |  |
| Height of conveying | H | m |  | measure |
| Length of conveying | L | m |  | measure |
| The angle of conveyor inclination | α | deg |  | calculate |
| Specific gravity of the material to convey* | γ | kg/m3 |  | in appen-dices |

*Instructor suggests the material to convey

5. Calculate the main parameters of the bucket conveyor and fill the table above.

| Parameter name | Unit of measurement | Formula | Value |
| --- | --- | --- | --- |
| Rate speed of the drive-shaft | rad/s | ω=πn/30 |  |
| Linear speed of the belt | m/s | v=Dω/2 |  |
| Pole distance | m | ρ=g/ω2 |  |
| Unloading method | - | - |  |
| Productivity of the conveyor | kg/s | Q=ψ γ і v/a |  |
| Energy-output ratio | - | Ке=360 Р/(QL) |  |

Calculations:

6. Show schemes of the loading and unloading of the bucket conveyors.

| loading scheme | unloading scheme |
| --- | --- |

Laboratory class conclusions with suggestions about improvement of bucket conveyors operation:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Part Two

Hoisting Machines

Ropes, Block-And-Tackles

Subject: the study and parameters calculation of ropes and block-and-tackles.

Purpose: to study the structure and main elements of the ropes and block-and-tackles; their main parameters.

Order of execution:

1. Draw the esquisses of the chains and ropes.

| Welded chain esquisse | Plate chain esquisse | Steel-wire rope esquisse (cross section) |
| --- | --- | --- |

2. Write down main characteristics of ropes and chains which are proposed by instructor.

|  | Welded chain | Plate chain | Steel-wire rope |
| --- | --- | --- | --- |
| Notation by the standart |  |  |  |
| Step of a chain, diameter of rope, mm |  |  |  |
| Failing load, H |  |  |  |
| Mass per unit length, kg/m |  |  |  |

3. Draw the block-and-tackle diagram that proposed by the instructor (straight-line diagram); show the forces.

| Parts of rope line: | Type of block-and-tackles: single or double |
| --- | --- |

4. Determine the biggest force in the drive line of the block-and-tackle which is caused by the weight of load Q.

Calculations:

5. Conduct an experimental study on the determination of the force in the drive line that is caused by the weight of the load. Determine the efficiency of the block-and-tackle.

Calculations:

Laboratory class conclusions with suggestions about improvement of block-and-tackles’ schemes, recommendations on enhancement of the drives i.e.:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Telpher

Subject: the study and parameters calculation of telphers.

Purpose: to study the kinematic scheme and main elements of the telphers; their main parameters.

Order of execution:

1. Draw the laboratory installation and show the main elements.

2. Draw the kinematic scheme of the telpher and show the main structural elements.

3. Draw the scheme of the hook assembly and specify its elements.

4. Fill the table below

| Parameter name | Notation | Unit of measurement | Value | Note |
| --- | --- | --- | --- | --- |
| Telpher movement along an I-beam | s | m |  | measure with tapeline |
| Duration of the telpher movement | t | s |  | measure with timer |
| The telfer speed | v | m/s |  | calculate |
| Diameter of the telfer wheel | d | m |  | measure with beam compass |
| Reduction ratio of the telfer drive | U |  |  | calculate |
| The number of teeth of the drive gear wheel | z1 | - |  | count |
| The number of teeth of the driven gear wheel | z2 | - |  | count |
| Reduction ratio of the exposed drive | Uex | - |  | calculate |
| Reduction ratio of the enclosed drive | Uen | - |  | calculate |
| Maximum load-lifting capacity | Qmax | kN |  | determine by a hook |
| The number of the hook | - | - |  | determine by a table in the laboratory |

5. Make a blueprint of the hook; specify main measures according to the standart.

Laboratory class conclusions with suggestions about improvement of telphers:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Jacks

Subject: the study and parameters calculation of jacks.

Purpose: to study the kinematic scheme and main elements of the jacks; their main parameters.

Order of execution:

1. Draw the kinematic scheme of the jacks and show the main structural elements.

| rack jack | screw jack |
| --- | --- |
| hydraulic jack | pneumatic jack |

2. Calculate the load-lifting capacity of the hydraulic jack. In calculation it is nessesary to take into account that allowable pressure in the hydraulic system is equal to 10 MPa.

Calculations:

3. Calculate the load-lifting capacity of the rack and the screw jacks. In calculation it is nessesary to take into account that allowable force of a worker is equal to 150…200 N.

Calculations of rack jack:

Calculations of screw jack:

5. Fill the table.

| Engineering data of jacks | Rack jack | Screw jack | Hydraulic jack |
| --- | --- | --- | --- |
| Model of the jack |  |  |  |
| Calculated load-lifting capacity, Н |  |  |  |
| Theoretically allowable load-lifting capacity, Н |  |  |  |
| Pillar travel, mm |  |  |  |
| Force on the lever, H |  |  |  |
| Efficiency, % |  |  |  |

6. Fill the table with suggestions about force gain for all the types of the jacks.

| Types of the jacks | Suggestions |
| --- | --- |
| Rack jack |  |
| Screw jack |  |
| Hydraulic jack |  |

Laboratory class conclusions about design of the jacks, their reliability, operational safety, maintainability:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Part Three

Manual Machines

Manual Machines

Subject: the study and parameters calculation of manual machines.

Purpose: to study the manual machines that are used in construction operations; their main parameters.

Order of execution:

1. Write down main elements of manual machines (use their instructions). Fill the table.

| Manual machine | Type |  | The list of the main elements |
| --- | --- | --- | --- |
| Electric hammer |  |  |  |
| Angle grinder |  |  |  |
| Screwdriver |  |  |  |
| Circular saw |  |  |  |
| Multi tool |  |  |  |
| Heat gun |  |  |  |

2. Fill the table and point the construction operations, where manual machines may be used.

| Manual machine | Type |  | The list of the construction operations |
| --- | --- | --- | --- |
| Electric hammer |  |  |  |
| Angle grinder |  |  |  |
| Screwdriver |  |  |  |
| Circular saw |  |  |  |
| Multi tool |  |  |  |
| Heat gun |  |  |  |

3. Write down the main parameters of manual machines. Fill the table.

| Parameter | Unit | Value of feature |
| --- | --- | --- |
| Electric hammer |  |  |
| Voltage | V |  |
| Power output | Wt |  |
| No-load speed | min-1 |  |
| Impact energy Hammerdrilling Chiselling | J J |  |
| Maximum drilling range in: Steel Wood Concrete | mm mm mm |  |
| Sound (noise) power | dB |  |
| Weight | kg |  |
| Angle grinder |  |  |
| Voltage | V |  |
| No-load/rated speed | min-1 |  |
| Power output | Wt |  |
| Wheel diameter | mm |  |
| Sound (noise) power | dB |  |
| Weight | kg |  |
| Screwdriver |  |  |
| Voltage | V |  |
| Battery Type | - |  |
| Power output | Wt |  |
| Max. torque (hard/soft) | Nm |  |
| Chuck capacity | mm |  |
| Maximum drilling capacity Wood Metal | mm mm |  |
| No-load speed 1st gear 2nd gear | min-1 min-1 |  |
| Weight (without battery pack) | kg |  |
| Circular saw |  |  |
| Max power out | Wt |  |
| Bevel stops | ° |  |
| Bevel capacity | ° |  |
| Depth of cut at 45° | In/Cm |  |
| Depth of cut at 90° | In/Cm |  |
| Blade diameter | In/Cm |  |
| Weight | kg |  |
| Multi tool |  |  |
| Power | Wt |  |
| Oscillation per minute | min-1 |  |
| Oscillation angle, left/right | ° |  |
| The noise level under working | dB |  |
| Overall length | mm |  |
| Net weight | kg |  |
| Heat gun |  |  |
| Power input | Wt |  |
| Air flow | l/min |  |
| Temperature range | °C |  |
| Weight | kg |  |

4. Fill the table with suggestions about manual machines safe operation.

| Manual machine | Type |  | The list of the suggestions about manual machines safe operation |
| --- | --- | --- | --- |
| Electric hammer |  |  |  |
| Angle grinder |  |  |  |
| Screwdriver |  |  |  |
| Buzz saw |  |  |  |
| Multi tool |  |  |  |
| Heat gun |  |  |  |

5. Use manual machines: to broke the concrete piece, to cut a piece of wood, to fasten two pieces of wood, to cut the piece of metal etc.

6. Fill the table with suggestions about ease of manual machines operation.

| Manual machine | Type |  | The list of the suggestions about ease of manual machines operation |
| --- | --- | --- | --- |
| Electric hammer |  |  |  |
| Angle grinder |  |  |  |
| Screwdriver |  |  |  |
| Buzz saw |  |  |  |
| Multi tool |  |  |  |
| Heat gun |  |  |  |

Laboratory class conclusions about design of the manual machines, their reliability, operational safety, maintainability:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Defended:______________________

Accepted:______________________

Content

| The order of the laboratory classes conducting | 3 |
| --- | --- |
| Part One. Conveying Machines | 4 |
| Belt Conveyor | 5 |
| Screw Conveyor | 8 |
| Bucket Conveyor | 11 |
| Part Two. Hoisting Machines | 15 |
| Ropes, Block-And-Tackles | 16 |
| Telpher | 18 |
| Jacks | 20 |
| Part Three. Manual Machines | 24 |
| Manual Machines | 25 |
| Content | 33 |